CMC & MSAT

# CC-AVI401-017
Technical position and
regulatory-impact brief

Proposed Helion HF-55 alternate overwrap versus the approved AVI-401 system

| Prepared for | Jonas Wei, Regulatory CMC Manager |
| --- | --- |
| Product / application | AVI-401 / APP-CN-AVI401-001 |
| Change | CC-AVI401-017 — qualify alternate overwrap supplier |
| Evidence cut-off | 08 February 2027 |
| Position | HOLD — technical equivalence not established; implementation not supported |
| Document status | Discussion brief; not a QMS approval, regulatory categorisation record, or implementation authorisation |

Purpose. Provide CMC/MSAT’s scientific position for the Regulatory CMC impact discussion, based on the approved BF-51 configuration and evidence currently available. Conclusions are deliberately separated from future evidence and formal decisions held in QMS/RIM.

# 1. Executive position

CMC/MSAT recommendation: do not approve, release, procure for commercial use, or implement HF-55 under CC-AVI401-017 at this time.

HF-55 is not a like-for-like supplier substitution. It changes the film construction and oxygen-barrier performance of a secondary packaging component that is explicitly part of AVI-401’s registered control strategy. At 23 °C / 65% RH, the supplier-reported oxygen transmission rate (OTR) is 3.8 versus 2.4 cm³/m²·day·bar for BF-51—approximately 58% higher oxygen transmission. The proposed film therefore moves in the adverse direction for the product’s fastest-moving degradation pathway, Impurity A (avericin N-oxide).

The present evidence supports only the approved Ilyssa BF-51 system. It does not establish HF-55 equivalence, an acceptable bridge to the existing 36-month shelf-life package, or process/seal suitability on Havenor’s line. The supplier expressly does not claim process equivalence or matched oxygen barrier, and no AVI-401 stability data with HF-55 are available.

Regulatory CMC should treat this as a substantive change to registered container-system information and the shelf-life control strategy—not as an administrative second-source update. Because APP-CN-AVI401-001 remains under review and the authority is currently asking specifically about BF-51 OTR, oxidation, worst-case pack and 36-month support, implementation must remain blocked pending a market-specific RIM assessment and any required prior authority clearance. The exact reporting category cannot be finalised from the current technical package.

## Decision statements for the discussion

Approved baseline remains CCS-AVI401-02 with Ilyssa BF-51 (PE/EVOH-32/PE, 51 µm). Only this configuration is supported by the filed stability narrative and approved validation reports.

HF-55 (MAT-OWR-005) may continue in controlled feasibility/qualification work, but must remain segregated and unavailable to ERP/MES commercial effectivity.

No shelf-life bridge should be claimed from BF-51 to HF-55 based on supplier OTR/WVTR data alone.

The open authority response and COM-AVI401-008 update should describe and support the registered BF-51 system. HF-55 should not be represented as approved or equivalent.

Reconsider the technical position only after the evidence gates in Section 6 are closed and independently reviewed.

# 2. Approved baseline versus proposed state

| Attribute | Approved AVI-401 system | Proposed HF-55 | CMC/MSAT interpretation |
| --- | --- | --- | --- |
| Supplier / material | Ilyssa BF-51 | Helion HF-55 / MAT-OWR-005 | Supplier and material both change. |
| Construction | PE / EVOH-32 / PE | PE / EVOH-44 / PE | Different EVOH grade; not formulation-identical. |
| Layer / total thickness | 20 / 5 / 26 µm; total 51 µm | 22 / 6 / 27 µm; total 55 µm | Different layer stack and +4 µm total thickness. |
| OTR, 23 °C / 0% RH | 0.8 cm³/m²·d·bar | 2.1 cm³/m²·d·bar | ~163% higher; materially weaker dry oxygen barrier. |
| OTR, 23 °C / 65% RH | 2.4 cm³/m²·d·bar | 3.8 cm³/m²·d·bar | ~58% higher at the available humidity-relevant comparison. |
| WVTR, 38 °C / 90% RH | 1.5 g/m²·d | 1.2 g/m²·d | HF-55 is ~20% lower for WVTR; this does not offset oxygen risk. |
| Seal layer / window | LDPE; 135–155 °C | LDPE; 132–152 °C | Partial overlap only; line settings and seal robustness not demonstrated. |
| Pouch cut-outs | 100 mL 140×180 mm; 250 mL 160×230 mm | Same nominal dimensions | Nominal fit is supportive but drawings remain outstanding in the change assessment. |
| Product contact | Non-contact, applied after fill/seal/terminal processing/inspection | Same intended use | Reduces direct leachables concern, but does not remove barrier, migration, particulate, seal or handling risks. |
| Product/process evidence | Filed stability through 18 months; approved current-system validation | No AVI-401 stability; no Havenor line run | Equivalence and shelf-life bridge are not established. |

Source: HFC-QN-2026-074 and filed Module 3.2.P.8. Ratios are simple comparisons of the reported values; no adjustment has been made for pouch area, seal area, headspace, storage distribution, or test-method uncertainty.

# 3. Scientific and product-quality assessment

## 3.1 Oxygen barrier is a critical functional attribute

The filed stability summary identifies BF-51 as the oxygen-barrier member of the marketed pack. AVI-401’s primary bag is polyolefin and is not a high-barrier film. Impurity A formation depends on residual headspace oxygen and oxygen permeation, and the 250 mL presentation (AVI401-R03) is identified as the more demanding oxidative case because of pack geometry and headspace-to-volume considerations.

The observed data are consistent with that mechanism: at 18 months/25 °C/60% RH, Impurity A is 0.35% for AVI401-R03 versus 0.30% and 0.29% for the 100 mL lots, against a 0.50% limit. At accelerated conditions, R03 reached 0.46% at six months; at intermediate conditions it reached 0.43% at 12 months. A film with higher OTR could increase oxygen ingress and steepen the Impurity A trend. The current data cannot quantify that effect for HF-55.

## 3.2 Existing shelf-life evidence does not create a bridge

STB-AVI401-002 is active and explicitly bound to CCS-AVI401-02/BF-51 and AVI401-SPEC-v03. The verified record currently contains results only through 18 months; the 24-month pull is scheduled for 01 March 2027 and remains pending. Existing registration-lot results therefore support the current system only.

As a risk-screening calculation—not a controlled shelf-life analysis—ordinary least-squares fits of the filed 0–18 month Impurity A values give the point estimates below. These reinforce the limited margin in the approved system and the need to avoid assuming that a weaker oxygen barrier can support 36 months.

| Lot / presentation | 18 m observed | 24 m linear point estimate | 36 m linear point estimate | Residual standard error |
| --- | --- | --- | --- | --- |
| AVI401-R01 / 100 mL | 0.30% | 0.372% | 0.533% | 0.0084% |
| AVI401-R02 / 100 mL | 0.29% | 0.371% | 0.525% | 0.0030% |
| AVI401-R03 / 250 mL | 0.35% | 0.446% | 0.636% | 0.0034% |

Screening method: unweighted linear regression of Impurity A at 0, 3, 6, 9, 12 and 18 months, fitted separately by lot; specification NMT 0.50%. No confidence bound, pooling, model qualification or rounding rule was applied. Regulatory use requires a controlled, independently reviewed analysis responding to HA-AVI401-IR-2027-014.

## 3.3 Other technical interfaces remain open

| Interface | Current evidence | Position / gap |
| --- | --- | --- |
| Seal integrity and process window | Supplier gives a recommended 132–152 °C window; Helion has not run Havenor’s line. AVT-VAL-202 supports only the approved configuration. | Execute approved line-fit and sealing-window protocol at justified edges; assess visual seal, package integrity and reproducibility. |
| Mechanical fit / distribution | Nominal pouch dimensions match; supplier drawings and qualification certificates are not present in the reviewed file set. | Obtain controlled drawings/certificates; perform fit, handling, print/label, packing and shipping configuration assessment. |
| Particulate / cleanliness | No representative HF-55 results available. | Define method and acceptance criteria; test representative lots and line-trial units. |
| Extractables / migration | Film is non-product-contact, but AVT-VAL-105 and AVT-VAL-201 cover the current system only. | Document a pathway-based CMC/toxicology risk assessment, including migration through/around the primary pack and effects of storage. |
| Supplier controls / incoming specification | Notification received; supplier qualification, quality agreement and incoming controls remain open. | Complete audit/qualification, change-notification controls, material specification and sampling plan. |
| Stability confirmation | No AVI-401/HF-55 data or approved bridge protocol. | Use both presentations and treat 250 mL as minimum worst-case for oxidation; protocol must measure Impurity A and define release/ongoing gates. |

# 4. Evidence available and evidentiary weight

| Evidence | Status at cut-off | What it supports | What it does not support |
| --- | --- | --- | --- |
| AVT-CMC-041 v01 | Draft; QA approval pending | Workstreams, risks and implementation prerequisites | Approval or technical equivalence |
| CC-AVI401-017 status export | technical_assessment; regulatory not started; implementation not authorised | Current workflow state and open gates | Validated or regulatory conclusion |
| HFC-QN-2026-074 | Controlled supplier notification; drawings/certificates referenced but absent from reviewed transmittal | HF-55 composition, dimensions, supplier test values, recommended seal window | Havenor process equivalence or AVI-401 shelf life |
| Filed 3.2.P.8 / AVT-CMC-038 v03 | Effective; sequence 0003 | Registered BF-51 role, 18-month data, commitment linkage | HF-55 bridge |
| AVT-STB-301 and LIMS exports | Approved / verified through 18 months | Current-system stability performance | 24- or 36-month outcome; alternate pack performance |
| AVT-VAL-201/202/203 | Approved for current system | Compatibility, closure/seal and administration-set evidence for CCS-AVI401-02 | Alternate material/supplier/configuration |
| HA-AVI401-IR-2027-014 | Official; response due within 60 days of 19 Jan 2027 | Active authority scrutiny of BF-51 OTR, oxidation, projection and worst case | Acceptance of HF-55 or change category |
| 24-month working planning files | Unsigned working files; pull not executed | Planning discussion only | Verified data, QC decision or regulatory evidence |

Data-governance note: this brief does not supersede QMS, RIM, LIMS, ERP, DMS or MES. Working files and exports must not be used as evidence of an approved state.

# 5. Regulatory CMC impact

## 5.1 Why the change is dossier-relevant

The filed container description names BF-51, its PE/EVOH-32/PE construction and 51 µm thickness; HF-55 changes those registered particulars.

The filed stability argument expressly relies on BF-51 oxygen-barrier performance. HF-55 changes a functional attribute linked to a specified degradation product.

The 36-month shelf life is not yet supported by completed long-term data, and the authority has requested projections and an explanation of BF-51 OTR and headspace control.

COM-AVI401-008 is linked to the approved container-system identity. Alternate-pack data and commitment traceability would need an explicit RIM/LIMS strategy.

## 5.2 Recommended regulatory stance

Regulatory CMC should open and complete the RIM impact assessment now, but record the current technical outcome as “insufficient evidence / implementation prohibited.” The change should be presumed to require prior regulatory clearance unless a market-specific assessment, based on a complete data package, justifies otherwise. It is not supportable as a notification-only administrative supplier addition on current evidence.

For APP-CN-AVI401-001 while under review: maintain BF-51 as the proposed/approved commercial configuration. Do not introduce HF-55 into filed Module 3 as an equivalent commercial alternative without authority strategy and sufficient supporting data.

For the open information request: answer the questions using the registered BF-51 system and controlled evidence. State when requested measurements are unavailable; do not use HF-55 supplier values to fill gaps in BF-51 evidence.

If the programme elects to pursue HF-55, prepare affected Module 3 updates (at least container-closure description and stability sections, plus specifications/controls and validation summaries as applicable) and determine submission timing/category per market.

Do not set ERP/MES effectivity, consume BF-51 stock, revise commercial instructions, or package saleable product with HF-55 before QA approval and required regulatory clearance are documented.

## 5.3 Interaction with the authority request

The authority’s questions increase—not decrease—the regulatory risk of the change. It has highlighted R03 at 0.35% Impurity A at 18 months, requested lot-specific projections and confidence bounds, asked for BF-51 OTR at relevant humidity, and requested confirmation of headspace control. A proposed film with 58% higher supplier-reported OTR at 65% RH cannot credibly be described as neutral until product-specific evidence establishes that shelf-life protection is maintained.

# 6. Minimum evidence and decision gates

| Gate | Minimum closure evidence | Owner(s) | Implementation criterion |
| --- | --- | --- | --- |
| 1. Material identity | Controlled HF-55 drawings, qualification certificates, composition/layer tolerances, test methods and CoA/specification | Packaging Engineering / Supplier Quality | Complete and approved |
| 2. Supplier controls | Qualification/audit, quality agreement, change-notification terms, incoming sampling/specification | Supplier Quality / QC | Supplier and material approved |
| 3. Barrier comparability | Comparative OTR across humidity relevant to long-term/accelerated storage; method comparability; pouch-level oxygen ingress or justified model; both sizes | Packaging Engineering / MSAT | Acceptance criteria met; model uncertainty addressed |
| 4. Process and seal | Line trial and seal-window qualification, including edge settings, integrity, visual/particulate, label and pack handling | Validation / Manufacturing / QC | Approved report; no unresolved deviations |
| 5. Product-contact pathway | Documented extractables/migration/toxicological risk assessment | CMC / Toxicology | Risk acceptable and approved |
| 6. Stability bridge | Approved protocol with BF-51 comparator, HF-55 confirmation lots, 100 and 250 mL, Impurity A focus, accelerated/long-term plan and predefined decision rules | QC Stability / CMC / Statistics | Sufficient data to support proposed shelf life and worst-case conclusion |
| 7. Regulatory | Market/application impact and category in RIM; affected dossier text; authority communication/clearance as required; COM-AVI401-008 linkage | Regulatory CMC | Documented clearance before first commercial use |
| 8. Quality/effectivity | Final AVT-CMC-041, QA change approval, aligned ERP/DMS/LIMS/RIM/MES effectivity, training, segregation, rollback and first-lot verification | QA / system owners | All authoritative systems agree |

# 7. Proposed path forward

Retain BF-51 as the sole authorised commercial overwrap and preserve stock/material segregation while the change is assessed.

Obtain the missing HF-55 drawings and qualification certificates; complete supplier qualification and define the incoming material specification before using test material on the line.

Agree a protocol-driven barrier and package study. At minimum, compare BF-51 and HF-55 at humidity conditions relevant to 25 °C/60% RH and stressed storage, include both pouch sizes, and evaluate oxygen ingress rather than relying only on flat-film OTR.

Execute line/seal qualification and product-specific stability work. Use the 250 mL presentation as the minimum oxidative worst case unless data justify otherwise; retain the 100 mL presentation to assess geometry/headspace effects.

In parallel, complete the controlled lot-specific BF-51 stability projection and authority response. Keep this work analytically separate from the HF-55 change package.

Reconvene CMC/MSAT, QC Stability, Packaging, Validation, QA and Regulatory CMC after gates 1–6. Regulatory CMC then finalises the market-specific reporting route and implementation prerequisites.

# 8. Overall risk conclusion

| Domain | Current rating | Basis | Residual position |
| --- | --- | --- | --- |
| Product quality / shelf life | High | Higher HF-55 OTR directly challenges control of Impurity A; current BF-51 margin to 36 months is unresolved. | Unacceptable for implementation until product-specific bridge demonstrates control. |
| Process / package integrity | Medium–High | Seal window differs and no Havenor line equivalence exists. | Qualification required. |
| Patient risk | Potentially significant, indirect | Failure would manifest as elevated degradation products or shortened shelf life. | Prevented by keeping HF-55 out of commercial use. |
| Regulatory compliance | High | Named registered component; open commitment and active authority questions. | Prior RIM decision and any authority clearance required. |
| Supply continuity | Not assessed as justification | Second source may improve resilience, but supply pressure is not scientific evidence. | May support priority, not lowered acceptance criteria. |

Bottom line: HF-55 is a potentially useful alternate-source candidate, but it is technically nonequivalent on the critical attribute currently available for comparison. The appropriate current disposition is HOLD / further evidence required, with BF-51 retained as the sole approved configuration.

# 9. Source records reviewed

CC-AVI401-017_Change_Status.xlsx — QMS change status export.

AVT-CMC-041, Alternate Overwrap Supplier Technical Assessment, v01, draft dated 08 Feb 2027.

HFC-QN-2026-074, HF-55 Film Notification, 04 Nov 2026.

AVT-CMC-038, 3.2.P.8 Stability — AVI-401, v03, effective 14 Oct 2026 / sequence 0003.

AVT-STB-301, AVI-401 interim stability report — 18 months, approved.

STB-AVI401-002_Long_Term_Results.xlsx and STB-AVI401-002_Study_Status.xlsx — verified study exports/status; next pull 01 Mar 2027.

AVT-SPEC-018 / AVI401-SPEC-v03, AVI-401 release and stability specification.

AVT-VAL-201, AVT-VAL-202 and AVT-VAL-203 — approved current-system container reports.

HA-AVI401-IR-2027-014, information request dated 19 Jan 2027.

AVT-SOP-095 v4.0, Technical Assessment of Component Changes; SOP-071 v5.0, Regulatory Change Assessment; AVT-SOP-074 v1.0, Post-Approval Change Reporting Categorisation.

# 10. Status and endorsement

This document records a recommended CMC/MSAT position for discussion. It is not an approval of CC-AVI401-017, a final regulatory reporting category, a shelf-life determination, or authorisation to implement HF-55. Formal conclusions must be entered and approved in the applicable QMS and RIM records.

| Role | Proposed disposition | Name / date |
| --- | --- | --- |
| CMC / MSAT technical owner | Concur / revise | ____________________________ |
| QC Stability reviewer | Concur / revise | ____________________________ |
| Regulatory CMC owner | Record market-specific assessment in RIM | Jonas Wei / __________________ |
| QA Product Lead | Quality decision remains pending | ____________________________ |