REGULATORY CMC

# Response to Information Request
HA-AVI401-IR-2027-014

Avericin Sodium Chloride Injection (AVI-401)

Application APP-CN-AVI401-001  |  Sequence referenced 0003

| Document status | INTERNAL WORKING DRAFT — NOT FOR SUBMISSION |
| --- | --- |
| Prepared for | Sofia Ortega, Head of Regulatory Affairs |
| Prepared by | Jonas Wei, Regulatory CMC |
| Evidence cut-off | 8 February 2027 |
| Authority letter date | 19 January 2027 |
| Response due | 20 March 2027 (60 days) |

DRAFTING CONTROL: Bracketed items and evidence gaps must be resolved before the response is signed or submitted.

Prepared against records available in the repository as of the evidence cut-off. No 24-month result is available or represented as available.

# Document control and response position

## Purpose

This working draft provides a point-by-point quality/Module 3 response to the National Medicinal Products Review Authority information request. It distinguishes statements supported by approved records from items that remain open. It is not an approved response and does not replace controlled source documents.

## Recommended regulatory position

| Topic | Position as of 8 February 2027 | Status |
| --- | --- | --- |
| P.7 leaf | Refile a corrected bookmarked/paginated leaf; no technical change. | Open — replacement leaf not available |
| Impurity A / shelf life | Separate lot regressions do not support 36 months. Recommend reducing the currently claimed shelf life to 18 months pending verified 24-month and later data. | Calculation ready; QC/statistical verification required |
| Overwrap / headspace | BF-51 OTR at 23 °C/65% RH is documented as 2.4 cm³/m²·day·bar. No registered-pack headspace oxygen control/measurement record was found. | Partial evidence |
| Accelerated / intermediate | 0.46% at 6 months remained within the 0.50% specification and was not treated as ICH Q1A(R2) significant change. STB-AVI401-003 was a standing Zone II precaution, not a corrective response. | Supported |
| LAB-2025-114 | Assignable cause and replacement result (0.14%) are documented; campaign-wide impact is not documented in the available event extract. | Partial evidence |
| Filter validation | Validation bounds are documented; production operating limits and conservative area-to-mass scaling are not demonstrated in the available records. | Partial evidence |
| Administration set | AVT-VAL-203 is approved but does not state requested duration, materials, AET, or last-fraction result. | Evidence gap |
| 24-month pull | Scheduled for 1 March 2027; not yet executed. Submit verified results under COM-AVI401-008. | Pending approved schedule |

## Submission-critical open items

Obtain and quality-control the corrected 3.2.P.7 leaf and its eCTD bookmarks/pagination.

Have QC Statistics independently reproduce the Impurity A regressions and approve the shelf-life position.

Provide controlled evidence for 100 mL and 250 mL headspace oxygen controls or explicitly confirm that no values were measured.

Obtain the approved process record establishing routine filtration time, product temperature, and filtered-bulk hold; document the test-to-production area-to-mass calculation.

Obtain the AVT-VAL-203 protocol/raw-data summary stating challenged infusion duration, set materials, analytical evaluation threshold, and last-fraction findings.

Obtain the complete LAB-2025-114 investigation or campaign assessment establishing whether any other sequence used the erroneous mobile phase.

Reconcile the unapproved Tomas Li planning workbooks, which misidentify presentation sizes and contain unverified projections; do not cite them as evidence.

# Proposed response text

The following section is drafted in applicant voice. Internal notes are shaded and are not intended for submission.

## Opening statement

Havenor Therapeutics Ltd. provides the following response to Information Request HA-AVI401-IR-2027-014 for APP-CN-AVI401-001. The response is based on the approved records and verified data available as of 8 February 2027. Where the requested information is not present in the available file, this is stated expressly. The 24-month long-term pull remains scheduled and no early pull has been performed solely for this response.

## 1. 3.2.P.7 bookmarking and pagination

A replacement 3.2.P.7 leaf with corrected bookmarks and pagination will be provided. The replacement will contain no new technical content.

INTERNAL: Attach and eCTD-QC the corrected P.7 leaf before submission. The application leaf/drawings were not available in the evidence set reviewed for this draft.

## 2. Impurity A trend, projection, worst-case presentation and shelf life

Impurity A (avericin N-oxide) was evaluated separately for each registration lot using the six verified long-term results at 0, 3, 6, 9, 12 and 18 months from STB-AVI401-002 (25 °C/60% RH). The acceptance criterion is not more than 0.50% area. Lots AVI401-R01 and AVI401-R02 are the 100 mL presentation; AVI401-R03 is the 250 mL presentation.

An ordinary least-squares linear model, y = a + bx, was fitted to each lot, where x is months and y is Impurity A (% area). The residual standard error (RSE) is √[Σ(yi−ŷi)²/(n−2)]. The one-sided 95% upper confidence bound shown below is for the fitted mean response: ŷ(x) + t0.95,4 × RSE × √[1/n + (x−x̄)²/Sxx], using t0.95,4 = 2.1318. No pooling was applied.

| Lot / pack | Observed 0–18 m (%) | Slope (%/month) | RSE (%) | 24 m fit / U95 (%) | 36 m fit / U95 (%) | U95 reaches 0.50 at |
| --- | --- | --- | --- | --- | --- | --- |
| R01 / 100 mL | 0.05, 0.10, 0.13, 0.17, 0.20, 0.30 | 0.013381 | 0.008409 | 0.372 / 0.394 | 0.533 / 0.568 | 31.3 months |
| R02 / 100 mL | 0.06, 0.10, 0.14, 0.18, 0.22, 0.29 | 0.012857 | 0.002988 | 0.371 / 0.378 | 0.525 / 0.538 | 33.2 months |
| R03 / 250 mL | 0.07, 0.11, 0.16, 0.21, 0.26, 0.35 | 0.015810 | 0.003381 | 0.446 / 0.455 | 0.636 / 0.650 | 26.8 months |

The fitted values at 24 months remain below the 0.50% limit for all lots. At 36 months, both the fitted mean and one-sided 95% upper confidence bound exceed the limit for every lot. The 250 mL lot AVI401-R03 has the steepest fitted increase, highest fitted result at 24 and 36 months, and earliest confidence-bound intersection (approximately 26.8 months); it therefore remains the worst-case pack in the present dataset.

Accordingly, the present dataset does not support the proposed 36-month shelf life. Because verified long-term observations currently extend only through 18 months, Havenor proposes an 18-month shelf life at 25 °C/60% RH at this stage. Study STB-AVI401-002 will continue on the approved 24-, 30- and 36-month schedule. The verified 24-month results will be submitted as the next stability update under COM-AVI401-008 and will be used to reassess the supported shelf life. No early pull is proposed.

Source: AVT-STB-301, pp. 1–2; STB-AVI401-002_Long_Term_Results.xlsx, sheets “Lots,” “Results,” and “Pull Schedule”; AVT-SPEC-018 v03, §2 and table of tests.

INTERNAL: Calculations above were generated for this draft and must be independently verified. The 18-month shelf-life proposal is conservative and requires Regulatory/Quality approval. Do not replace observed data with the unapproved projections in Tomas Li’s planning workbook.

## 3. Registered overwrap oxygen transmission and headspace oxygen

The registered secondary overwrap is BF-51, PE/EVOH-32/PE. The available supplier file reports an oxygen-transmission rate of 2.4 cm³/m²·day·bar at 23 °C/65% RH (and 0.8 cm³/m²·day·bar at 23 °C/0% RH). The 65% RH value is the available humidity-relevant datum nearest the 25 °C/60% RH long-term condition; no conversion to 25 °C/60% RH has been made. Source: HFC-QN-2026-074, p. 1, comparison table.

The available approved records do not state a numerical residual-headspace-oxygen limit, a nitrogen-flush set point, or measured residual headspace oxygen for either the 100 mL or 250 mL registration fills. Havenor therefore cannot provide those values from the present file. [Insert controlled manufacturing record citation if located; otherwise retain this disclosure in the final response.]

BF-51 remains the registered overwrap. The proposed alternate HF-55 (MAT-OWR-005) is not authorized for implementation; change control CC-AVI401-017 remains at technical assessment with regulatory assessment not started and implementation blocked.

INTERNAL: Do not assert that the stability trend itself proves headspace control. Obtain batch/master-record evidence or retain the explicit “not in file” statement.

## 4. Accelerated Impurity A, intermediate study, and LAB-2025-114

The 6-month accelerated Impurity A result of 0.46% area for AVI401-R03 at 40 °C/75% RH remained within the registered limit of not more than 0.50%. Assay remained within 1.3% of initial for all three lots, and the approved interim report records no specification failure. On that basis, the result was not treated as a “significant change” under ICH Q1A(R2).

STB-AVI401-003 (30 °C/65% RH) was placed as a standing precaution for Zone II markets in the registration plan, not as the response to an accelerated significant-change finding. It therefore should not be characterised as the “complete response” to such a finding. Available intermediate results through 12 months comply with AVI401-SPEC-v03. Source: AVT-STB-301, p. 1, §3; AVI-401_Accelerated_and_Intermediate_Results.xlsx, sheets “Studies” and “Results.”

For LAB-2025-114, the assignable cause was a mobile-phase preparation error confirmed against the balance printout; system-suitability tailing was outside method limits. The original Impurity B run for STB-AVI401-001 / AVI401-R02 / 6 months was invalidated, and the reportable confirmatory result from the retained sample was 0.14% area (limit NMT 0.30%). Source: AVI-401_Accelerated_and_Intermediate_Results.xlsx, sheets “Laboratory Events” and “Results.”

The event extract available for this response does not contain the sequence list or a documented campaign-wide assessment showing whether the same mobile phase was used for any other reported sequence. Havenor therefore cannot confirm campaign-wide non-impact from that extract alone. [Insert approved LAB-2025-114 investigation citation and conclusion if obtained.]

INTERNAL: A closed status alone is insufficient to answer the authority’s campaign-scope question. Obtain the complete investigation/audit-trail review.

## 5. Filter validation boundary and production coverage

AVT-VAL-104 challenged the sterilised PVDF membrane with eight-hour recirculating AVI-401 contact at 30 °C and 2.1 bar differential pressure after two autoclave cycles. AVT-VAL-105 used a 24-hour exaggerated extraction at 40 °C and an eight-hour AVI-401 process simulation at 30 °C. Both identify the 47 mm hydrophilic PVDF disc (effective test area 13.8 cm² in the approved AVT-VAL-105-P protocol) as representative of the 254 mm production cartridge and support MAT-FLT-022/FT-AVI401-02 within the stated validated boundaries. Source: AVT-VAL-104, pp. 3 and 5; AVT-VAL-105, pp. 3 and 5; AVT-VAL-105-P v01, pp. 4 and 6.

The approved records available for this response do not state the numerical routine production filtration time or product-temperature limit. They also do not provide the membrane-area-to-product-mass values or calculation needed to demonstrate that the disc scale-down is conservative for extractables. Accordingly, those confirmations cannot be made from the present file. [Insert approved master production record/process-validation citation and the scale-down calculation when obtained.]

The controlled process summary states that approved hold-time controls apply, but does not state their duration. No available record demonstrates that filtered bulk is permitted to be held longer than eight hours. Any contact or hold beyond eight hours, temperature above 30 °C, pressure above 2.1 bar, or changed effective area would be outside the cited validation boundary and would require assessment before use. Source: AVT-MFG-PCS-012, p. 1; AVT-VAL-104, pp. 14–15; AVT-VAL-105, pp. 30–31.

INTERNAL: Final response requires positive confirmation from an approved executed/master process record, not inference from the validation report.

## 6. Administration-set compatibility

AVT-VAL-203 confirms in outline that three independent executions met its approved acceptance criteria and that the challenged contact duration was the maximum routine duration plus 20%. However, the approved report available for this response does not identify the numerical infusion duration, the administration-set materials, the analytical evaluation threshold, or analyte results for the last collected fraction. Havenor therefore cannot provide the requested values from AVT-VAL-203 as filed. Source: AVT-VAL-203, pp. 4–5 and 7.

[Insert the controlled protocol/raw-data citation and state: (i) infusion duration; (ii) tubing, drip chamber, connector and any plasticiser materials; (iii) AET and units; and (iv) whether any leachable above the AET was observed in the final fraction. If the underlying record does not contain these data, retain an explicit statement that they were not measured.]

INTERNAL: This point is not submission-ready. Do not infer “no leachable above AET” from the generic pass conclusion.

## 7. 24-month long-term stability update

The 24-month long-term pull for STB-AVI401-002 is scheduled for 1 March 2027 for lots AVI401-R01, AVI401-R02 and AVI401-R03 at 25 °C/60% RH. As of 8 February 2027, the pull has not occurred and no 24-month result is available. Havenor will provide the verified 24-month results as the next stability update under commitment COM-AVI401-008. No earlier pull will be performed solely to answer this information request. Source: STB-AVI401-002_Study_Status.xlsx, sheets “Study” and “Pull Schedule.”

INTERNAL: Do not promise inclusion with the 20 March response unless the complete result set is verified and released. Existing working plans show impurity testing after 20 March and CCI in April and are not approved.

## Closing statement

Havenor respectfully submits the above information in response to the Authority’s questions. The applicant will continue STB-AVI401-002 on the approved schedule and will provide verified 24-month data under COM-AVI401-008.

# Appendix 1 — Calculation details for Impurity A

Input data are the reportable STB-AVI401-002 Impurity A values in the controlled long-term results workbook. All calculations use unrounded coefficients; displayed values are rounded only for presentation.

| Parameter | AVI401-R01 | AVI401-R02 | AVI401-R03 |
| --- | --- | --- | --- |
| Presentation | 100 mL | 100 mL | 250 mL |
| n / df | 6 / 4 | 6 / 4 | 6 / 4 |
| Intercept, a (% area) | 0.05128571 | 0.06214286 | 0.06685714 |
| Slope, b (% area/month) | 0.01338095 | 0.01285714 | 0.01580952 |
| Residual standard error (%) | 0.00840918 | 0.00298807 | 0.00338062 |
| R² | 0.992533 | 0.998972 | 0.999130 |
| Fitted mean at 24 m (%) | 0.372429 | 0.370714 | 0.446286 |
| One-sided 95% UCB at 24 m (%) | 0.393532 | 0.378213 | 0.454769 |
| Fitted mean at 36 m (%) | 0.533000 | 0.525000 | 0.636000 |
| One-sided 95% UCB at 36 m (%) | 0.568403 | 0.537580 | 0.650233 |
| UCB/specification intersection | 31.310 m | 33.172 m | 26.778 m |

## Interpretive limitations

Each lot has only six timepoints through 18 months; projections materially beyond the observed interval are uncertain.

The confidence bound is for the fitted mean response, consistent with the stated shelf-life regression approach; it is not an individual future-observation prediction interval.

No model pooling was used because the authority requested each registration lot and the 250 mL lot shows a visibly steeper trend.

The linear fit is descriptive; the 24- and 36-month values are projections, not measured stability results.

The displayed intersection is not itself a recommended labelled shelf life. The proposed 18 months is anchored to the verified long-term period currently observed.

# Appendix 2 — Evidence register

| Source | Status / date | Use in draft | Precise location |
| --- | --- | --- | --- |
| HA-AVI401-IR-2027-014 | Issued 19 Jan 2027 | Authority questions and deadline | pp. 1–2 |
| AVT-STB-301 | Approved; current 18-month report | Long-term results; accelerated/intermediate context | pp. 1–2 |
| STB-AVI401-002_Long_Term_Results.xlsx | Active study | Lot/presentation mapping; reportable data; schedule | Lots; Results; Pull Schedule |
| STB-AVI401-002_Study_Status.xlsx | Active | 24-month pull state | Study; Pull Schedule |
| AVT-SPEC-018 v03 | Effective 20 Jan 2025 | Impurity A limit and methods | §2, test IMPA |
| AVI-401_Accelerated_and_Intermediate_Results.xlsx | Current data extract | Accelerated/intermediate values; event extract | Results; Laboratory Events |
| HFC-QN-2026-074 | Supplier notification 4 Nov 2026 | BF-51 OTR comparator | p. 1 comparison table |
| CC-AVI401-017_Change_Status.xlsx | Technical assessment | HF-55 implementation status | Change; Required Evidence |
| AVT-VAL-104 v01 | Approved | Chemical-compatibility boundary | pp. 3, 5, 14–15 |
| AVT-VAL-105 v01 / AVT-VAL-105-P v01 | Approved | E&L boundary and test area | Report pp. 3, 5, 8, 10, 30–31; protocol pp. 4, 6 |
| AVT-MFG-PCS-012 | Effective reference | Approved configuration/lifecycle control | pp. 1–2 |
| AVT-VAL-203 v1.0 | Approved 16 Jan 2025 | Administration-set report outline and identified gaps | pp. 4–5, 7–9 |

# Appendix 3 — Internal approval checklist

| Gate | Owner | Required closure | State at cut-off |
| --- | --- | --- | --- |
| Regulatory content | Regulatory CMC | Accept response positions; resolve brackets | Open |
| Statistical calculation | QC Statistics | Independent reproduction and approval | Open |
| Shelf-life decision | Quality / Regulatory | Approve 18-month proposal or alternative justified position | Open |
| P.7 replacement | Publishing / Regulatory Ops | Validated bookmarks and pagination | Open |
| Headspace evidence | MSAT / Manufacturing | Controlled limit/measurement or confirmed absence | Open |
| LAB-2025-114 scope | QC | Approved investigation and campaign-impact conclusion | Open |
| Filter process coverage | MSAT / Validation | Routine limits and area-to-mass calculation | Open |
| Administration-set details | Validation / CMC | Protocol/raw-data facts requested in point 6 | Open |
| 24-month update | QC Stability | Execute, verify, release, then submit under commitment | Scheduled 1 Mar 2027 |